# CFS RFID | Electrical Wiring Reference

Seeed XIAO ESP32-C6 + RC522 + ST7789 1.54-inch TFT + buzzer

Firmware v0.3.7 TEST  •  26 October 2026  •  3.3 V logic

# 1. Complete connection schedule

| Controller pin | Connects to | Function |
| --- | --- | --- |
| XIAO 3V3 | RC522 3.3V; TFT VCC | 3.3 V power rail |
| XIAO GND | RC522 GND; TFT GND; buzzer GND | Common ground |
| D8 / GPIO19 | RC522 SCK; TFT SCL | Shared SPI clock |
| D10 / GPIO18 | RC522 MOSI; TFT SDA | Shared SPI data out |
| D9 / GPIO20 | RC522 MISO | SPI data in |
| D3 / GPIO21 | RC522 SDA / SS / CS | RC522 chip select |
| D2 / GPIO2 | RC522 RST | Reader reset |
| D4 / GPIO22 | TFT CS | Display chip select |
| D5 / GPIO23 | TFT DC | Display data/command |
| D6 / GPIO16 | TFT RST | Display reset |
| D1 / GPIO1 | Buzzer/speaker signal | Existing signal output |
| GPIO3; GPIO14 | No external connection | Reserved antenna RF switch |

# 2. Shared-SPI wiring schematic

SEEED XIAO ESP32-C6
              +--------------------------------+
  3V3 --------|----+---- RC522 3.3V           |
              |    +---- TFT VCC              |
  GND --------|----+---- RC522 GND            |
              |    +---- TFT GND              |
              |    +---- Buzzer GND           |
  D8 GPIO19 --|----+---- RC522 SCK            |
              |    +---- TFT SCL              |
  D10 GPIO18 -|----+---- RC522 MOSI           |
              |    +---- TFT SDA              |
  D9 GPIO20 --|--------- RC522 MISO           |
  D3 GPIO21 --|--------- RC522 SDA/SS/CS      |
  D2 GPIO2 ---|--------- RC522 RST            |
  D4 GPIO22 --|--------- TFT CS               |
  D5 GPIO23 --|--------- TFT DC               |
  D6 GPIO16 --|--------- TFT RST              |
  D1 GPIO1 ---|--------- Buzzer signal        |
              +--------------------------------+
  TFT BL: backlight circuit -- verify module before wiring.

# 3. Electrical and integration notes

TFT header order: As marked on the photographed ST7789 PCB: GND, VCC, SCL, SDA, RST, DC, CS, BL. "SDA" here means SPI MOSI, not I2C SDA.

TFT backlight: The test arrangement discussed uses BL at 3V3, but the backlight resistor/current-limit implementation on this exact module is unverified. Check the module circuit and backlight current before permanent direct connection; use an appropriate current-limited driver if necessary.

Shared SPI: Both devices share SCK and MOSI. Only the RC522 uses MISO. Each device has a separate CS line; deselect both before starting SPI transactions.

Antenna selection: Firmware selects XIAO onboard ceramic Wi-Fi antenna (external U.FL connector damaged). GPIO3 and GPIO14 are dedicated to the XIAO RF antenna switch and must not be used for peripherals.

Power: Power the RC522 from 3.3V, not 5V. Check total TFT + RC522 + buzzer load against the XIAO 3.3V supply capability. Use common ground.

RFID pins: The RC522 pin marked SDA is its SPI chip-select (SS/CS) in this project. RC522 IRQ is deliberately unconnected.

Buzzer: D1/GPIO1 is signal only; buzzer power wiring depends on its type. Do not connect a high-current speaker directly to a GPIO. Use a transistor/driver where required.

Deployment protection: This is a wiring reference only. Existing printer SSH/SCP sync, database and CFS logic should remain unchanged.

Validation: The firmware pin mapping was checked against v0.3.7 TEST source. Physical solder joints and backlight circuitry cannot be verified from firmware.

# 4. Firmware pin declarations

RFID: SCK 19 / MISO 20 / MOSI 18 / CS 21 / RST 2
TFT: CS 22 / DC 23 / RST 16
Buzzer: GPIO1
Reserved antenna RF switch: GPIO3 and GPIO14